# Annex II – Core TSOs general measures and action plan to avoid future cross-zonal capacity reductions

Q3 2025

Table of Contents

Introduction	2

1.	CEPS	4

2.	DAVinCy TSOs	4

2.1.	APG	4

2.1.1	Overview	4

2.1.2	Conclusion	5

2.2.	German TSOs	5

2.3.	TENNET TSO BV	5

3.	ELES	5

4.	ELIA	5

5.	HOPS	5

6.	MAVIR	6

7.	PSE	6

8.	RTE	7

9.	SEPS	8

10.	Transelectrica	8

# Introduction

This annex contains the required information for each Core TSO that has applied capacity reductions for its CNECs in more than 1% of ID CC MTUs of the analysed quarter as described per Article 18(12).

# CEPS

During the third quarter of 2025, the Czech transmission system operator (ČEPS) applied the Individual Validation Adjustment (IVA) procedure a total of 246 times in 144 distinct MTUs.

These adjustments were exclusively related to the V404, V497, V443 and V444 interconnection lines located in the CZ–PL–SK corner of the grid—an area consistently subject to high transit flow pressure.

A significant portion of the IVA applications stemmed from the fact that these lines were already heavily utilized in the day-ahead market, despite meeting the 70% cross-zonal capacity criterion. This pre-existing usage limited operational flexibility and, in several cases, triggered the need for IVA due to insufficient margin for secure operation.

In this period (Q3/2025), one of the two 400 kV cross-zonal connections between the CZ and AT bidding zones was undergoing planned maintenance, and the remaining one was also subject to a temporary outage related to maintenance and partial upgrades. As a consequence, there was a period during which no 400 kV connection between the two bidding zones was available. During this time, all market flows in the highly in-demand direction from Western to Southeastern Europe were redirected to parallel transmission paths—primarily through the 220 kV cross-zonal lines between CZ and AT and through all available cross-zonal lines along the CZ–SK border. The resulting grid situation is illustrated in the accompanying figure (with lines in planned outage marked in blue and lines with IVA applications marked in red).

The increased frequency of IVA in Q3 reflects the operational challenges in a highly utilized grid region, while also underscoring ČEPS's proactive approach in adapting to forecast limitations and preserving grid security under pressure.

Starting in November, SEPS implemented its seasonal “winter” limit on the V404 Nošovice–Varín line, as it is not yet prepared to apply DLR, which would otherwise allow for higher utilization of this corridor. Since the beginning of the month, we have not observed—nor do we expect—any significant reductions.

Picture: Grid situation (blue – lines in planned outage, red – lines with IVA applied)

# DAVinCy TSOs

## APG

## German TSOs

## TENNET TSO BV

# ELES

General measures to avoid cross-zonal capacity reductions in the future, as per Article 18(11)(b) of ID CCM:

Improvement in congestion management – we will continue to improve the quality of our inputs for CORE ID CC in order to avoid unnecessary IVA application in case of errors in inputs files.

Network development and optimisation.

Detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future as per Article 18(10)(h)(i) of ID CCM
   
 In Q3 of 2025, ELES applied reduction in 1,72 % of MTU. 71% of these reductions were applied on CNECs on AT-SI border (Kainachtall – Maribor 1&2, Maribor-Cirkovce 1&2). The remaining 29% of applied reductions were applied on CNECs on the HR-SI border (Divaca-Pehlin, Podlog-Zerjavinec and Krsko-Tumbri 1&2). The main reason for the above reductions at ELES lies in the fact, that we are in the cross-road of two CCR, which both aim to maximise capacities in order to fulfil 70% criteria in DA. Often, maximisation of the capacities in Italy North CCR have negative effect on the RAM of Slovenian elements in CORE CCR. This is mostly due to the fact, that we have a PST on the Slovenian – Italian border, that is used to maximise Italy North NTC values. Maximising capacities in DA often translates to low capacities available for Intraday.

Our plan to improve the situation consists of the following:

Additional training of operators and improvement of the local validation tool in order to improve the process and improve stability and reliability of the tool

Analysis will be performed on accuracy of validation tool (e.g. comparing the flows considered during the validation and realised flows). Based on the result of the analysis, the validation tool reliability margin will be adjusted in order to decrease the level of IVA application.

Network development and optimisation - For AT-SI border, we are in the process to obtain and install a static serous synchronous compensator (SSSC) in Podlog substation (Q4 2027 or early 2028) in order to be able to redistribute and relieve the flows on the elements Kainachtal – Maribor and Obersielach- Podlog. For the Divaca – Pehlin line, we are still investigation different possibilities to increase capacities (SSSC or high temperature lines).

In Q4 2025 ELES plans to implement DTR on all SI-HR tie-lines, that will presumably increase its Fmax in most of MTUs (expected positive impact on Divača-Pehlin tie-line)

Improve the outage planning coordination.

# ELIA

# HOPS

General measures to avoid cross-zonal capacity reductions in the future, as per Articles 18(10)(h)(i) and 20(11)(b) of the ID CCM

General measures include, but are not limited to:

· Network development and optimization

The goal is to increase the transmission capacity and reduce grid congestion. The measures to achieve these goals include strengthening and optimizing the existing network and the development of new infrastructure.

· Improvements concerning congestion management

Core CCR coordinated improvements with coordinated actions to increase cross-zonal capacities (for example improvements of the outage planning coordination in order to increase flexibility of the grid). Introduction of additional effective remedial actions should help to relieve the congestion and therefore allows to reduce the number of IVA application. Also, inclusion of third countries could open further opportunities for HOPS (with planning process and implementation of remedial measures). Unscheduled allocated flows coming from commercial exchanges outside the Core CCR (Fuaf) has a strong impact on HOPS grid.

Detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future

In the analysed quarter (Q3 2025), HOPS applied reduction to around 6,52% MTUs. For most MTUs, the reductions are applied to:

· TL 220kV Zakucac - Mostar (0,14 % MTUs, or around 1,78% of times of all HOPS applied reductions)

· TL 400kV Zerjavinec - Tumbri (0,05% MTUs, or around 0,60% of times of all HOPS applied reductions)

· TL 400 kV Ernestinovo – Pecs 1 (0,09% MTUs, or around 1,18% of times of all HOPS applied reductions)

· TL 400 kV Ernestinovo – Pecs 2 (0,09% MTUs, or around 1,18% of times of all HOPS applied reductions)

· TL  220kV Pehlin - Divaca (5,62% MTUs, or around 73,37% of times of all HOPS applied reductions)

· TL   220kV Zerjavinec - Podlog (0,41% MTUs, or around 5,33% of times of all HOPS applied reductions)

· TL  400kV Tumbri - Krsko 1 (0,45% MTUs, or around 5,92% of times of all HOPS applied reductions)

· TL  400kV Tumbri - Krsko 2 (0,82% MTUs, or around 10,65% of times of all HOPS applied reductions)

Applied reductions on network element are mostly low (less than 2,18% of Fmax), while for a few MTUs during July (BD20250705 and BD20250711), August (BD20250806) and September (BD20250927) higher values are applied on TL 400kV Tumbri - Krsko 1 and 2 due to unsolvable overloads in the relevant grid area caused by unavailability of grid elements in the surrounding area, additionally under the influence of high exchanges between Core and non-Core countries (impact of uncoordinated flows coming from third countries, mainly from the direction of RS and BA). It is important to emphasize that energy needs increased during Q3 2025 in SEE with significant flows in the network towards SEE, which is why some reductions were applied during Q3 2025 to maintain a secured network. Such reductions are planned to be solved by developing and optimising the transmission network.

In Q2 of 2025, HOPS installed HTLS conductors on TL 220 kV Brinje – VE Pađene that increases its maximum admissible power flow and improves available capacities.

Also, following the ongoing investigation of various options for increasing capacity for the TL 220 kV Pehlin – Divača, it is planned to start installing HTLS conductors on TL 220 kV Pehlin – Divača in Q4 2026.

Improvements are also expected with upcoming important processes such as coordinated validation capacities and Regional Operational Security Coordination.

# MAVIR

# PSE

General measures to avoid cross-zonal capacity reductions in the future, as per Art. 20(14)(b) of ID CCM:

PSE is taken under consideration: long-, medium- and short-term measures to prevent capacity reduction.

Generally, the main source of improvements will be grid developments as foreseen in the Grid Development Plan.

In medium PSE is investigating dynamic monitoring of the lines, which increase the line rating.

As the short-term measures, PSE implemented parametrization of the validation tool, which potentially leading to avoiding application of low IVA values (so that IVAs will be less frequent).  Additional propose was to include in individual validation topological remedial actions.

In some cases the IVA was implemented in specific maintenance situation, this will be only temporary and additional investigation are not foreseen,

Detailed report describing how such deviations are expected to be alleviated and solved in the future.

In the analysed quarter (Q3 2025), PSE applied reduction of ~1,2% MTUs = 27 distinct MTUs with applied reductions. In total it was 2417MW of IVA applied as outcome of individual validation.

In the analysed quarter (Q3 2025), for most MTUs, the reductions are applied to:

CNE PST in Mikulowa station PSE applied 1022MW of IVA (~42,3% of total reductions applied as outcome of individual validation tool), mainly due to influence of planned outages in Mikulowa substation and high F0Core.

Wielopole - Nosovice PSE applied 453MW of IVA (~18,7% of total reductions applied as outcome of individual validation tool). Applied reductions results mainly from the planned outage in considered area.

Detmarovice – Dobrzen PSE applied 253MW of IVA (~10,5% of total reductions applied as outcome of individual validation tool), mainly due to influence of planned outages on internal line and high F0Core.

Tucznawa - Byczyna PSE applied 192MW of IVA (~7,9% of total reductions applied as outcome of individual validation tool), mainly due to high F0Core.

Rzeszow - Khmelnytskyi PSE applied 106MW of IVA (~4,4% of total reductions applied as outcome of individual validation tool). Applied reductions results mainly from the planned outage in considered area.

Krosno Iskrzynia – Rzeszow PSE applied 104MW of IVA (~4,3% of total reductions applied as outcome of individual validation tool). Applied reductions results mainly from the planned outage in considered area.

CNE Krajnik-Vierraden PSE applied 97MW of IVA (~4,0% of total reductions applied as outcome of individual validation tool), mainly due to influence of planned outages on internal line.

Kopanina - Liskovec PSE applied 62MW of IVA (~2,6% of total reductions applied as outcome of individual validation tool), mainly due to high F0Core

Mikulowa AT1 PSE applied 58MW of IVA (~2,4% of total reductions applied as outcome of individual validation tool), mainly due to influence of planned outages in Mikulowa substation.

Mikulowa AT2 PSE applied 58MW of IVA (~2,4% of total reductions applied as outcome of individual validation tool), mainly due to influence of planned outages in Mikulowa substation.

Polaniec - Tarnow PSE applied 12MW of IVA (~0,5% of total reductions applied as outcome of individual validation tool). Applied reducions results mainly from the planned outage in considered area.

# RTE

# SEPS

The reductions were mainly applied during the summer months, which were characterized by significant heat waves combined with high forecasted transit flows. In addition, Q3 2025 was affected by planned outages of multiple overhead lines, which further contributed to the need for reductions where IVA was applied.

In the analysed quarter (Q3 2025), SEPS applied reductions in approximately 1.45% of MTUs, with a total curtailed volume of 9,939 MW. For most MTUs, the reductions were applied on the following tie-lines:

TL 400 kV [CZ–SK] Nošovice – Varín [DIR] [SK]

TL 400 kV [SK–SK] Veľký Ďur – Levice 1 [DIR]

TL 400 kV [SK–HU] Levice – Göd [DIR] [SK]

General measures to avoid cross-zonal capacity reductions in the future

A planned reinforcement of the Nošovice – Varín tie-line is expected to increase the maximum current (Imax) from 1740 A to 2000 A by 2029, which should reduce the IVA volume applied on this element.

For the CNE Veľký Ďur – Levice 1 and Levice – God the following solutions is planned:
 The aim of the project is to connect the V492 Veľký Ďur – Horná Ždaňa line to the 400 kV switchyard in the Levice substation, to modify the connection of the V491 Veľký Ďur – Levice line in the 400 kV switchyard in the Levice substation, and to interconnect the V490 Veľký Ďur – Levice and V449 Levice – SR/HU state border (Göd) lines, including adjustments to secondary equipment in the related SEPS substations. The project is currently scheduled for completion by Q2 2029.

The national ten-year development plan also includes additional planned reinforcements.

# Transelectrica

In addition to being part of the Core CCR, Transelectrica is also part of SEE CCR with RO – BG border having operational processes for the First Intraday capacity calculation since October 2021 and Second Intraday capacity calculation since October 2022. Moreover, there are three non-EU borders for which there is no coordinated capacity calculation.

Regarding the capacity calculation process, the Core ID CCM allows TSOs to correct cross-zonal capacity for reasons of operational security during the validation process individually and in a coordinated way according to Article 18 (2): “Each Core TSO shall validate and have the right to decrease the RAM for reasons of operational security during individual validation. [...] IVA may reduce the RAM only to the minimum degree that is needed to ensure operational security, and only after all the expected available costly and non-costly remedial actions pursuant to Article 22 of the SO Regulation are considered”.

For the period 20250701 – 20250930, Transelectrica applied an Individual Validation Adjustment (IVA) on the following CNEs, for a total of 10.51% of MTUs. In the graph below the average IVA per CNE is represented for the timestamps where reductions were applied.

OHL 400 kV Portile de Fier – Djerdap is a tieline between Romania and Serbia, both countries having tielines with Bulgaria and Hungary. Because of this, the power flows on OHL 400 kV Portile de Fier – Djerdap are highly impacted by the cross-border exchanges between RO-BG, RO-RS, RO-HU.

General measures to avoid cross-zonal capacity reductions in the future, as per Articles 18(10)(h)(i) and 20(11)(b) of the ID CCM.

Development of the transmission grid

The main measure to reduce overloads in the capacity calculation process is to finalize investment projects with cross-border impact:

New OHL 400 kV Portile de Fier – Resita and TIE 400 kV Resita – Pancevo circ. 2 have been commissioned in mid of November 2024. TIE 400 kV Resita – Pancevo circ. 1 was commissioned in February 2025.

New Autotransformer 400/220 kV in Rosiori substation to be commissioned in 2028;

400 kV OHL Resita – Timisoara – Sacalaz to be commissioned in 2027;

400 kV d.c. OHL Timisoara – Arad to be commissioned in 2028;

Increasing the transmission capacity on the OHLs 220 kV Urechesti – Targu Jiu Nord – Paroseni – Baru Mare – Hasdat to be commissioned in 2028;

Increasing the transmission capacity on the OHL 220 kV d.c. Portile de Fier – Resita to be commissioned in 2028;

400 kV TIE Nadab – Graniceri circ. 2 to be commissioned in 2028;

400 kV TIE Portile de Fier – Djerdap circ.2 to be commissioned in 2029;

400 kV TIE Oradea Sud – Josza to be commissioned in 2030.

Outage planning coordination

Relevant assets are coordinated in the Outage coordination regions. This coordination has the scope of avoiding simultaneous disconnection of multiple elements with cross-border impact, taking into consideration the system security as a first step. Lately it has been observed that the finalization of the Yearly Maintenance Plan is highly impacted by the level of cross-border capacities provided to the market. Thus, it became a necessity to coordinate the outage of relevant assets also from the perspective of the capacity calculation processes.

Implementation of regional coordinated processes for security analysis and redispatching and countertrading processes

Results of the intraday capacity calculation process come with a lot of uncertainties, mainly due to the input data, volumes of capacity allocated and available redispatching. Measures to decrease the need of application of IVA include the consideration of internal redispatching. Due to the proximity to real-time, multiple power plants cannot be considered. Also, depending on the location of the congested elements, there may be none, one or multiple generators that can help decreasing the overload, but not necessarily avoiding them altogether.

It is thus mandatory to have security analysis implemented at regional level before real-time with coordinated means of reducing the observed overloads.

At Core CCR level these processes are still under implementation.